- Hazard is a system state or set of conditions that, together with a particular set of worst-case environmental conditions, will lead to an accident.

In this article, there are some hazards, and I listed it below.

- Radiation overdose -> Patients in hospital received higher radiation doses than they expected and it can cause severe injury.

- Radiation beam Spill -> Radiation beam for small target spills outside the target and may damage another spot.

- Hidden incorrect settings for radiation beam -> Operators could not see this incorrect setting directly because a metal tray on which the cone is mounted hids the jaws, though the settings should have been displayed on a computer screen, according to people who have worked with this device.

- A Failure to Communicate -> Error founded when treatment data moves through a chain of devices and software.

- Risks of Mix and Match systems -> Radiation safety experts say the retrofitted devices made up of different companies’ products present a special challenge.

- Calibration tool errors for SRS treatments -> Using wrong calibration tools can cause systematic dose errors.

- Operator (Human) errors -> Wrong setup, Process misunderstanding and no double-checking can cause operator errors.

- Lack of safety features -> SRS unit at Evanston lacked certain safety features, including those that might have prevented radiation from leaking outside the cone.

- Weak reporting -> Dr. Holmberg says there is no effective way now of sharing the information or learning in a systematic way and reporting needs to be transparent and mandatory.

Automatic Mode fails to trigger

Remote Manual Mode fails to trigger

Valves fail to open

FCU does not issue Release Water Command

Water Not Released

Water Released but Ineffective

Failure to suppress fire

FCU fails to receive and process manual command

Operator fails to issue manual release command

Heat detector fails to detect heat above threshold

Smoke detector fails to detect smoke above threshold

(a)

| Data Item/Event | Failure Mode (generic, e.g., “Absent data” or “Incorrect Logic/Event) | Description of Failure Mode (as instantiated in this system) | Effect (optionally, sub-divide into 3 columns: “Local Effects,” “System Effects,” & “Global Effects”) | Criticality (reference Lecture 2 severity levels, or alternate definitions) |
| --- | --- | --- | --- | --- |
| Alarm activation command | Absent Data | Alarm activation command didn’t work when incubator temperature is not in desired range | Nurse did not get alerted, which could lead danger by unsafe temperature | Catastrophic |
| Alarm activation command | Incorrect Data | Alarm activation command worked in wrong temperature | False alarm or missed alarms comes out. | Catastrophic |
| Alarm activation command | Timing of Data Wrong | Alarm activation command work too late after temperature goes up or down. | Delayed nurse intervention. Which could lead danger by unsafe temperature | major |
| Alarm activation command | Duplicated Data | Alarm works multiple time for a single event | It may make nurse confused | minor |

(b)

| Data Item/Event | Failure Mode (generic, e.g., “Absent data” or “Incorrect Logic/Event) | Description of Failure Mode (as instantiated in this system | Effect (optionally, sub-divide into 3 columns: “Local Effects,” “System Effects,” & “Global Effects”) | Criticality (reference Lecture 2 severity levels, or alternate definitions) |
| --- | --- | --- | --- | --- |
| Heat Source turns OFF | Halt/Abnormal Termination | Heat doesn’t go up even though it is under the range | It could cause temperature drop | major |
| Heat Source turns OFF | Omission | Heat keeps going up even though temperature is in the range | May cause overheating, which could lead baby in danger | major |
| Heat Source turns OFF | Incorrect Logic/Event | Heat is controlled by wrong logic | It may cause instability | minor |
| Heat Source turns OFF | Timing/Order | Heat turns on and off in a wrong timing | Fail to maintain temperature in the range | minor |